Program ADJ2LINE   Version 9314

by:       David H. Ransom, Jr.

7130 Avenida Altisima

Rancho Palos Verdes, CA  90274

RPV Astronomy BBS

2400 to 14400 baud

(310) 541-7299

RPV Hotline BBS

1200 and 2400 baud

(310) 544-8977

Program ADJ2LINE will allow the user to convert a set of 2-line orbital

elements from one epoch to another. For reasonable accuracy, the two epochs

should be relatively close to each other.

The screen sample below illustrates the operation of ADJ2LINE.

In this case, a set of 2-line elements was available pre-launch but the

launch was delayed 5 minutes and 1 second from the planned time. (Rather

than enter an original time of "0" and a new time of "00:05:01", the actual

times could have been used instead.)

The program first requests the name of the 2-line orbital elements file

which includes the satellite of interest. Enter the full file name using

upper or lower case letters. The file name will be automatically

capitalized. If no filetype is included, the filetype will default to

".TXT".

Next, enter the name or partial name of the satellite. If you simply press

ENTER, the name will default to "STS".

The 2-line orbital elements file is searched until a satellite name is

found whose starting letters match the satellite name just input. The

elements are displayed for approval. Press ENTER to accept that satellite,

or any other letter to continue the search.

Enter the original launch date in the form "mm/dd/yy" or press ENTER to

signify that the original launch date and the new launch date are the same.

Enter the new launch date in the form "mm/dd/yy" when requested.

Now enter the original launch time in the form "hh:mm:ss" (or zero)

followed by the new launch time (or the delay time). Note that the new

launch time may be BEFORE or AFTER the original launch time; the program

makes the necessary adjustments. Launch dates and times may be in any

convenient time zone but must be consistent for the "original" and "new"

entries.

ADJ2LINE will display the original EPOCH and RIGHT ASCENSION OF THE

ASCENDING NODE (RAAN) followed by the new calculated values and the

adjusted 2-line elements (with checksums corrected as required).

Now enter the full file name to which the new 2-line orbital elements

should be written. If no filetype is specified, a filetype of ".TXT" will

be automatically included. If you simply press ENTER, the file NEW2LINE.TXT

will be written. If the requested output file already exists, you will be

asked if you wish to overwrite that file. Press "Y" or ENTER to overwrite

the file, or press "N" to enter a different filename.

Version 9241 -- October, 1992

-----------------------------

-There was a minor bug in Version 9117 which caused the RAAN to be shifted

to the left if the angle was less than 100 degrees. This resulted in 5 or 6

digits in the fractional part rather than 4. The error has been corrected.

Version 9308 -- February, 1993

------------------------------

-Corrected a bug that caused the Epoch to be processed incorrectly if one

or more spaces were present following the year. This error could only occur

in the early part of the year when the day number was less than 100; for

example, some 2-line elements might show an Epoch time of 12:00 UTC on

February 20, 1993 as "93 51.50000000" instead of "93051.50000000". Zeroes

are now automatically substituted for those spaces during processing and in

the output file.

-Changed the date input method to specify the full original and new dates

instead of "day difference". The day difference method had problems if the

new date (or time) was BEFORE the original date. If the two dates are the

same, the user may simply press ENTER, then enter the times (or time

difference) as before.

-The program tests to see if the requested output file exists and requests

permission before overwriting it.

Version 9314 -- March, 1993

---------------------------

-Corrected a bug which caused problems in the RAAN format if the new time

and date were BEFORE the original time and date, making the RAAN negative

and generally disrupting that field.

SCREEN SAMPLE RUN:

------------------

Program ADJ2LINE   Version 9314

This program will allow the user to convert a set of 2-line

elements from one epoch to another. For reasonable accuracy,

the two epochs should be relatively close to each other.

Enter 2-line Filename:  STS53R02.TXT

Enter Satellite Name:   STS

The 2-line elements read for this satellite are:

STS 53          37.1 17.4 23.8  1.1

1 00053U          92337.56011644  .00015200  00000-0  25599-3 0    25

2 00053  56.9987 150.2015 0003435 263.1784  96.8439 15.65964837    29

Press ENTER to ACCEPT this satellite,

or any other key to continue searching:  YES

Enter dates as 'MM/DD/YY' or press ENTER if dates are the same

Enter Original Date:    (dates are the same)

Enter times as 'HH:MM:SS', 'HH:MM.MM', or 'HH.HHHH'

Enter Original Time:    0

Enter New Time:         00:05:01

Original Epoch:         92337.56011644

Original RAAN:            150.2015

Estimated New Epoch:    92337.56360024

Estimated New RAAN:       151.4591

The new 2-line elements for this satellite are:

STS 53          37.1 17.4 23.8  1.1

1 00053U          92337.56360024  .00015200  00000-0  25599-3 0    24

2 00053  56.9987 151.4591 0003435 263.1784  96.8439 15.65964837    21

Enter new 2-line Filename [NEW2LINE.TXT]:  NEW2LINE.TXT

File NEW2LINE.TXT already exists! Overwrite? [Y,n]:  YES